About the Project

System profile and recommended prerequisites

Aera’s core path is simple: the ESP32 senses, the cloud processes and stores, and client applications present the results. This profile identifies the main technologies and the background useful for following their implementation.

Technical profile

| System | IoT environmental-monitoring prototype linking an ESP32 sensor station, cloud services, and web/mobile clients. |
| --- | --- |
| Sensing | DHT22 measures temperature and humidity; an MQ-9 analog signal is used to estimate carbon monoxide. |
| Firmware and setup | Arduino C++ on ESP32 handles sampling, device identity, status indicators, and captive-portal setup for local 2.4 GHz Wi-Fi. |
| Telemetry | About every five seconds, readings and the device ID are sent as JSON over HTTPS to the Aera Cloud REST API. |
| Cloud and data | Python/FastAPI validates and processes readings; PostgreSQL stores durable records. Redis is optional. |
| Applications | React web dashboard and Expo/React Native Android/iOS apps use HTTP APIs for requests and WebSockets for live updates. |
| Capabilities | Pair a station, view current readings and history, and consult regional Ambient data. AI guidance/email are optional integrations. |

Architecture and learning value

Aera separates responsibilities across the device, cloud, and client layers. The ESP32 focuses on sensing and reliable telemetry; the backend provides a shared place to validate, process, and store readings; the apps turn that data into current and historical views. Because web and mobile clients use the same API, device communication and user-facing features can evolve without giving each interface direct access to the database.

This structure makes the project useful for studying more than an air-quality interface: it connects embedded programming, network protocols, server-side validation, persistent data, and live updates in one working system. Optional services such as caching and AI guidance extend the experience, while remaining separate from the core sensor-to-cloud data path.

Recommended prerequisites

•  Programming fundamentals — A basic understanding of variables, conditions, and functions is useful; C++ experience is helpful, not essential.

•  Hardware and networking — Introductory familiarity with sensors, microcontroller inputs, Wi-Fi, and HTTP will help. The report explains Aera’s own setup.

•  Development environment — Be comfortable opening project files in an editor and following terminal instructions. Prior React, FastAPI, or PostgreSQL experience is not required.

Measurement note: MQ-9 values are estimates and no particulate sensor is fitted. Aera is an educational prototype, not a certified detector or safety alarm.

CONTENT

| CHAPTER 1 — PROJECT OVERVIEW AND TECHNICAL FOUNDATIONS | 01 |
| --- | --- |
| 1.1 Introduction | 01 |
| 1.2 Project Rationale | 01 |
| 1.3 Aim and Objectives | 01 |
| 1.4 Scope and Intended Use | 02 |
| 1.5 How the System Works | 02 |
| 1.6 The Four Main Components | 03 |
| 1.6.1 Mobile application | 03 |
| 1.6.2 Web dashboard | 03 |
| 1.6.3 Cloud backend and data services | 03 |
| 1.6.4 ESP32 sensor node | 03 |
| 1.7 Languages, Frameworks, and Development Tools | 04 |
| 1.8 How the Parts Communicate | 04 |
| 1.9 Capabilities, Boundaries, and Responsible Interpretation | 05 |
| 1.10 Reading the Rest of This Book | 05 |
| CHAPTER 2 — HOW AERA WORKS: FROM SENSOR TO SCREEN | 06 |
| 2.1 Overview of the Operating Flow | 06 |
| How the Main Path Moves | 06 |
| 2.2 Power-Up, Identity, and Network Readiness | 08 |
| 2.3 Sampling and Preparing a Telemetry Record | 08 |
| 2.4 Sending the Reading and Identifying the Device | 09 |
| 2.5 Cloud Processing, AQI, and Alert Decisions | 09 |
| 2.6 Live State, Historical Storage, and Device Pairing | 10 |
| 2.7 Delivering Updates to the Mobile and Web Clients | 10 |
| 2.8 The Ambient Grid Path | 11 |
| 2.9 Boundary of This Chapter | 11 |
| CHAPTER 3 — TOOLS, COMPONENTS AND INSTALLATION GUIDE | 12 |
| 3.1 Aera’s Workspaces and Their Toolchains | 12 |
| 3.1.1 Software Tools: Purpose, Download and Learning | 12 |
| Shared foundations | 12 |
| Web dashboard | 12 |
| Mobile application | 12 |
| Cloud backend and data | 12 |
| Firmware and platform build tools | 12 |
| 3.1.2 Physical Components | 14 |
| 3.2 Obtain the Repository and Prepare the Workstation | 14 |
| Shared workstation prerequisites | 15 |
| 3.3 Prepare the Web Dashboard | 15 |
| Install the locked web dependencies | 15 |
| 3.4 Prepare the Python Backend | 16 |
| Create the isolated environment | 16 |
| Create private backend configuration | 16 |
| 3.5 Prepare the Mobile Application | 17 |
| Install the JavaScript workspace dependencies | 17 |
| Android native requirements | 17 |
| iOS native requirements | 17 |
| 3.6 Prepare the ESP32 Firmware Toolchain | 18 |
| 3.7 Prepare Databases, Optional Services, and Environment Files | 19 |
| 3.7.1 PostgreSQL: required for durable backend history | 19 |
| 3.7.2 Redis and integrations: optional | 19 |
| 3.7.3 Docker Compose: optional combined service setup | 19 |
| 3.7.4 Create local environment files | 19 |
| 3.8 Installation Readiness Checklist | 20 |
| Shared and repository access | 20 |
| Web and mobile | 20 |
| Backend and firmware | 20 |
| CHAPTER 4 — CODING AND ENVIRONMENT: KEY IMPLEMENTATIONS | 22 |
| 4.1 Reading the Code as One Project | 22 |
| 4.2 ESP32 Firmware: Files, Pins and Key Routines | 22 |
| 4.2.1 Firmware file structure | 22 |
| 4.2.2 ESP32 signal and pin diagram | 23 |
| 4.2.3 Power distribution and common ground | 24 |
| 4.2.4 Imports and hardware declarations | 24 |
| 4.2.5 Identity and sensor preparation | 25 |
| 4.2.6 Sampling, transmission and device response | 26 |
| 4.3 Backend: Application, APIs and Persistence | 26 |
| 4.3.1 Application entry point and imports | 28 |
| 4.3.2 Environment settings and database sessions | 28 |
| 4.3.3 Telemetry contract and processing service | 29 |
| 4.3.4 Account authentication and access tokens | 29 |
| 4.3.5 Pairing a device to an account | 30 |
| 4.3.6 Historical readings and live WebSocket updates | 30 |
| 4.3.7 AI chat, recommendations and guardrails | 31 |
| 4.4 Web Dashboard: Imports, APIs and Live State | 31 |
| 4.4.1 Imports, page composition and hooks | 33 |
| 4.4.2 API client, configuration and authentication | 33 |
| 4.4.3 Service endpoints and live state | 33 |
| 4.4.4 Device discovery, claiming and source selection | 34 |
| 4.4.5 AI assistant and per-tab chat history | 34 |
| 4.4.6 Custom web favicon | 35 |
| 4.5 Mobile App: Imports, APIs and Local State | 35 |
| 4.5.1 Native entry point, providers and screen imports | 36 |
| 4.5.2 API client and service methods | 37 |
| 4.5.3 AsyncStorage and reading history | 37 |
| 4.5.4 Device pairing and native history | 37 |
| 4.5.5 Assistant session and ambient context | 37 |
| 4.5.6 App icon and splash artwork | 38 |
| 4.6 How the Four Implementations Connect | 39 |
| CHAPTER 5 — TESTING, DEPLOYMENT AND PACKAGING | 40 |
| 5.1 Verification sequence and safe test data | 40 |
| 5.1.1 Project folders and command locations | 40 |
| 5.2 ESP32 firmware: compile, upload and inspect | 41 |
| 5.2.1 Prepare and verify the sketch | 41 |
| 5.2.2 Upload and inspect device startup | 43 |
| 5.2.3 Confirm telemetry before moving on | 43 |
| 5.3 Backend: migrations, tests and local health check | 43 |
| 5.3.1 Isolate the test database | 44 |
| 5.3.2 Start the API and check readiness | 45 |
| 5.3.3 Confirm the ingestion path | 45 |
| 5.4 Web dashboard: test, build and preview | 45 |
| 5.4.1 Install dependencies and confirm API settings | 46 |
| 5.4.2 Build and serve the production files | 46 |
| 5.4.3 Review results before continuing | 47 |
| 5.5 Optional toolchain: Docker Compose | 47 |
| 5.5.1 What Docker Compose runs | 47 |
| 5.5.2 Install and configure from the repository root | 47 |
| 5.5.3 Build and start the local services | 48 |
| 5.5.4 Read the output and know what was verified | 48 |
| 5.6 Mobile app: configuration, tests and native projects | 49 |
| 5.6.1 Confirm API configuration and validate the Expo project | 49 |
| 5.6.2 Regenerate the native iOS and Android projects | 50 |
| 5.6.3 Review the generated native-project changes | 50 |
| 5.6.4 iOS: configure signing, select a destination, and build | 52 |
| 5.6.5 iOS: install and trust the app on a physical iPhone | 53 |
| 5.6.6 Android: assemble a release APK | 55 |
| 5.6.7 Android: install the app from GitHub Releases | 57 |
| CHAPTER 6 — SYSTEM OUTPUTS AND USER INTERFACES | 59 |
| 6.1 ESP32 Sensor Node: Hardware and Serial Output | 59 |
| 6.1.1 Physical Assembly and Sensor Connections | 59 |
| 6.1.2 Serial Communication and Telemetry Output | 59 |
| 6.2 Aera Web Dashboard: Ambient Grid and Live Readings | 60 |
| 6.3 Aera Mobile Application Interface and Operation | 61 |
| Mobile Application Walkthrough | 61 |
| 6.3.1 Launch | 61 |
| 6.3.2 Authentication | 61 |
| Sign in | 61 |
| Workspace setup | 61 |
| Password recovery | 61 |
| 6.3.3 Ambient and Custom Hardware Sources | 63 |
| 6.3.4 Dashboard guidance and trends | 64 |
| 6.3.5 Atmospheric AI assistant | 64 |
| 6.3.6 Telemetry history | 65 |
| 6.3.7 Operator settings | 65 |
| REFERENCES | 67 |
| Glossary and Acronyms | 69 |